King Fahd University of Petroleum & Minerals

Physics Department

PHYS 308 (Term 211)

First Major Exam (1 hour 30 minutes, 15/15)

The solution must contain all the detailed steps and calculations

Monday, October 25, 2021

Always take the value of 0.7 V for Silicon

Problem 1. (2.5/15)

The following circuit uses identical diodes with IS = 1.5x10-16 A, n = 1 and VT = 0.025 V. Use the exponential model to determine the value of Vo when IB = 3.2 mA and R = 1k.

Solution.

ID=IR2=1.6 mA

VD=nVT lnIDIS=0.025 ln1.6×10-31.5×10-16=0.75 V

Vo=2VD+IBR=1.5+3.2=4.7 V

Problem 2. (2.5/15)

The following circuit shows a voltage regulator with a Zener diode (VZ0 = 5V, rz = 50 ), a resistor R = 500 , and a load resistor RL = 1k.

- Write an expression for ∆V0/∆VS.

- Find the variation in output voltage if the source voltage VS varies by  500 mV.

a)

∆VO∆VS=RL     rZRL     rZ+R

b)

∆VO=∆VSRL     rZRL     rZ+R=±500 mV47.6547.6=±43.5 mV

Problem 3. (2.5/15)

For the circuit below:

- Find the output DC voltage.

- Determine the ripple factor in percentage.

- Is the capacitor used appropriate for the required ripple factor? Explain

- Determine the load current.

Solution.

See the solution in homework 1.

Problem 4. (2.5/15)

Determine the Q-point for the circuit below and draw the dc load line. Find the maximum peak value of base current for linear operation. Assume DC = 200.

Problem 5. (2.5/15)

Determine VCE and IC in the voltage-divider biased transistor circuit of the figure below if DC = 100.

Solution.

VB≅R2R1+R2VCC=5.6 kΩ15.6 kΩ10V=3.59 V

VE=VB-VBE=3.59 V-0.7 V=2.89 V

IE=VERE=2.89 V560 Ω=5.16 mA

IC≅IE=5.16 mA

VC=VCC-ICRC=10 V-5.16 mA1.0 kΩ=4.84 V

VCE=VC-VE=4.84 V-2.89 V=1.95 V

Problem 6. (2.5/15)

Determine whether the transistor in the figure below is in saturation.

Assume VCE (sat) = 0.2 V.

Solution.

First, we determine IC(sat).

ICsat=VCC-VCE(sat)RC=10 V-0.2 V1.0 kΩ=9.8 V1.0 kΩ=9.8 mA

Now, we see if IB is large enough to produce IC(sat).

IB=VBB-VBERB=3 V-0.7 V10 kΩ=2.3 V10 kΩ=0.23 mA

IC=βIB=500.23 mA=11.5 mA

This shows that with the specified , the base current is capable of producing an IC greater than IC(sat). Therefore, the transistor is saturated.