Problem 1

For the common-emitter (CE) amplification circuit below:

- Determine the bias current IE. 2 points

- Determine Rin(total) seen by the source (Vin). 2 points

- Determine the voltage gain from base to collector without the load (RL). 2 points

- Determine the voltage gain from base to collector when a load (RL=5 k) is connected to the output. 2 points

- What will happen to the voltage gain when the bypass capacitor is removed? 1 point

- Why is a partially bypassed emitter resistor used? 1 point

Note: In AC analysis, use the simplified T-model of BJT at room temperature.

- 2 points

- 2 points

Rintot=R1   R2   Rin(base)

Rinbase = βacre' =160 × 6.98 = 1.12 k

re'=25 mVIE=0.025 V3.58 mA=6.98

Rin(tot)=1122 kΩ+16.8 kΩ+11.12 kΩ=921.3

- 2 points

A=VcVb=RCIcre'Ie=RCIere'Ie=RCre'=1 k6.98 =143.3

- 2 points

A'=RC   RLre'=RC×RLRC+RLre'=0.83 k6.98 =118.9

- 1 point

The voltage gain will be reduced. The effect of RE is to decrease the ac voltage gain. The new gain A will be in this case:

A=RCre'+RE

- 1 point

A partially bypassed emitter resistor used (with RE partially unpassed), the gain is much less dependent on re'.

A=RCRE1 without load or RC   RLRE1 (with load)

Problem 2. (4/10)

The amplifier circuit uses an op-amp with finite gain A of 10. Find Vout for Vin = 1V.

Take R1 = 1 k and R2 = 2 kΩ.

Vout=AVd

Vout= -1.54

Problem 3. (3/10)

The circuit of a noninverting summing amplifier is shown below. The circuit consists of two inputs V1 and V2. Calculate the output Vout of the amplifier as a function of the two inputs V1 and V2.

Use superposition principle to find the contribution of V1 to the output voltage (V2 = 0).

V+=32+3V1=0.6 V1

Vout=1+91V+=10×0.6 V1=6 V1

Use superposition principle to find the contribution of V2 to the output voltage (V1 = 0).

V+=22+3V2=0.4 V2

Vout=1+91V+=10×0.4 V2=4 V2

Vout=6 V1+4 V2

Problem 4. (3/10)

Calculate the voltage gain, current gain, power gain, and input resistance.

i1=1V-01 k=1 mA

i2=i1=1 mA

Vout=0-1 mA×10 k=-10 V

iL=Vout1 k=-10 V1 k=-10 mA

iout=iL-i2=-10 mA-1 mA=-11 mA

Voltage gain = Vout1 V=-10 V1 V=-10 (or 20 dB)

Current gain = iLi1=-10 mA1 mA=-10 (or 20 dB)

Power gain= Vout1 V×iLi1=-10×-10=100 (or 20 dB)