# Ch.03.Neutron Distributions in Energy

1	Introduction	2

2	Nuclear Fuel Properties	2

3	Neutron Moderators	4

4	Neutron Energy Spectra	5

4.1	Fast Neutrons	5

4.2	Neutron Slowing Down	5

4.3	The Slowing Down Density	5

4.4	Energy Self-Shielding	6

4.5	Thermal Neutrons	6

4.6	Fast and Thermal Reactor Spectra	7

5	Energy-Averaged Reaction Rates	8

5.1	Fast Cross Section Averages	9

5.2	Resonance Cross Section Averages	9

5.3	Thermal Cross Section Averages	9

6	Infinite Medium Multiplication	11

7	Problems	12

# Introduction

Multiplication Factor:K=Number of neutrons in i+1th generationNumber of neutrons in ith generation

- Two simplifications used in Ch03:

  - All neutrons are born instanteously (no delayed fission neutrons, ch05)

  - Infinite size reactor with no leakage or a spatial variation (ch06-7)

- Here, you are moving neutrons population from fast to thermal using energy considerations only

# Nuclear Fuel Properties

- Neutron energy range is restricted to 0.001 eV- 10 MeV

  - 10 MeV high limit is based on fission neutrons spectrum

  - 0.001eV low limit is based on:

    - Maxwell Boltzmann distribution of neutrons

    - High absorption cross section of low energy neutrons

- Fission process can happen:

  - Fissile nuclei (U-235 and Pu-2390) can fission with neutrons having energy over this entire range

  - Fertile nuclei U-238 can only fission when the absorbed the neutron with energy>1MeV

- Fission to absorption ratio eta η is determined by:

| ηE=νΣf(E)Σa(E) | 3.3 |
| --- | --- |

  - Nu: ν is the average fission neutrons created if the fission happens at E

- ηE must be larger than 1 to make up for:

  - Non-fission absorption

  - Leakage from the reactor

- After each absorption, the nucleus will either settle to a ground state by emitting a gamma ray or break up as fission:

| ΣaE=ΣγE+Σf(E) | 3.4 |
| --- | --- |

- Using the macroscopic cross section is the correct way unless you consider a single isotopic material. If a single material is considered, then 3.3 can be rewritten as:

| ηE=νσf(E)σa(E) | 3.5 |
| --- | --- |

  - This is useful to see how a pure U-235 or Pu-239 behave:

- Most power reactors use enriched fuels with an (atomic)enrichment ratio (e tilde) given by:

| e=NfiNfi+Nfe | 3.6 |
| --- | --- |

  - The fission to absorption ratio eta is given by the following formula

| ηE=eνσffiE+1-eνσffe(E)eσafiE+1-eσafe(E) | 3.7 |
| --- | --- |

  - Note that the fertile material can also go through fission although much less and with an energy threshold

- Plotting fission to absorption ratio eta for enriched fuel, gives a clear picture:

- For 20% enriched fuel:

  - Thermal region: 0.0001eV to 1 eV has large eta factor of 2

  - Fast region: 1MeV and above eta is high and even higher than thermal region

  - Intermediate region: 1eV-1MeV, has low ratio which cannot sustain chain reaction

- This behavior is the origin of designing:

  - Thermal power reactor:

    - keep neutron population in the thermal region

    - this is done by effective moderation of neutrons (born fast) toward thermal region

    - Need: 1) moderators 2) coolant

  - Fast reactors:

    - Reduce moderation to a minimum by eliminating low z nuclei

    - Need: 1) coolant which is not moderating

  - There are no intermediate energy reactors.

# Neutron Moderators

- Neutrons are generated at high energy distribution (chi distribution in ch02:χ(E))

- Table 1 list some key temperature land marks and shows that fission neutrons are, in some way, extremely hot particles

- Moderators are materials that reduces energy of neutrons coming from fission reaction toward thermal energy

- Three key criteria must be considered in selecting moderators:

  - Large slowing down energy decrement (xi (ksee), ξ): Need low atomic number

    - ξ=lnEE'=∫ln⁡(EE') p(E→E') dE'

    - pE→E'dE'=11-αEdE',αE<E'<E0otherwise

    - ξ≈1+α1-αlnα, α=A-12A+12

  - Large scattering cross section, (sigma scattering, Σs)

  - Small relative absorption cross section, (sigma absorption, Σa) in thermal energy region

- Table 2 shows three common moderating nuclei.

- Heavy water can be used to moderate even natural uranium

- Light water necessitates use of enriched fuel due to thermal absorption

- Selected moderating material affect the overall size of the reactor

- Boron 10 satisfies large decrement and large scattering cross section but has huge absorption cross section

  - It forbids using it as moderator

  - It is used as a control (poison) to stop the chain reaction.

- Coolants: (in all cases low absorption cross section)

  - Thermal reactor:

    - The same as moderators

    - Different from moderators

  - Fast reactor: must have minimal moderating effect:

    - high A nuclei: low energy decrement

    - low scattering cross section

    - low absorption cross section

Table 1: Fission neutrons are born fast and very hot! Mono-atomic gas energy E=32kT, k is Boltzmann constant.

| # | Case | Temperature (K) | Temperature (eV) |
| --- | --- | --- | --- |
| 1 | Core of Earth | 5700 K | 0.737 eV |
| 2 | Surface of the Sun | 5778 K | 0.747 eV |
| 3 | Core of the Sun | 15×106K=15 million K | 1.94 keV |
| 4 | ITER Tokamak Fusion reactor | 150×106K=150 million K | 19.4 keV |
| 5 | Most probable fission neutron energy | 5.80×109K=5.8 billion K | 0.75 MeV |
| 6 | Average fission neutron energy | 1.55×1010K=16 billion K (≈1000X sun's core temp) | 2.0 MeV |

Table 2

# Neutron Energy Spectra

- Neutrons at any energy level are either born from fission into energy E or scattered from the general flux ϕ(E) into energy E:

| ΣtotEϕE=∫ΣspE'→EϕE'dE'+χESf''' | 3.15 |
| --- | --- |

## Fast Neutrons

- In the fast (high) energy range, neutrons flux can be assumed to be totally dominated by just born fission process:

| ϕE≈χESf'''/ΣtotE | 3.16 |
| --- | --- |

## Neutron Slowing Down

- Scattering can only decrease the energy of neutrons for energies greater than 1 eV

- Up scattering can happen only bellow 1 eV (where energy of the neutron is increased due to scattering)

## The Slowing Down Density

- Any neutron produced by fission that is not absorbed at a higher energy must slow down past that energy (kind of check point)

| qE=Number of neutrons slowing down past energy E/s/cm3 | 3.17 |
| --- | --- |

- Thus, for energies above 1 eV:

| qE=-E∞ΣaE'ϕE'dE'+E∞χE'dE'sf''', E > 1.0 eV | 3.18 |
| --- | --- |

- Bellow 0.1MeV, the integral is approximately unity:

| qE≈-E∞ΣaE'ϕE'dE'+sf''', 1.0 eV< E < 0.1 MeV | 3.19 |
| --- | --- |

- In this formula, there are two terms:

  - The first is proportional to absorption cross section

  - The second term is constant and energy independent

- Thus, after a few steps of spectral manipulation, we reach the following approximation, where q in the intermediate region is a constant dependent on the material

| q=1+α1-αlnαC | 3.25 |
| --- | --- |

| ϕ(E)=qξ ΣsEE | 3.26 |
| --- | --- |

- This flux is “one over E” flux

  - Higher E less flux

  - Lower E higher flux (1/x behavior)

- This is the general behavior of the flux (expected behavior)

- The slowing down decrement, ksee or ξ, is obtained using atomic weighting between the fuel and the moderator. As shown in equation 3.27:

## Energy Self-Shielding

- U-238 has resonance absorption peaks with higher than the general behavior absorption curve

- These absorption peaks cause depression or self-shielding in the flux per energy spectrum around the absorption peaks

- Such phenomena only exist in the epithermal region and is not observed in thermal or fast regions

Link

## Thermal Neutrons

| ΣtotEϕE=0E0ΣspE'→EϕE'dE'+E0∞ΣspE'→EϕE'dE' | 3.15 |
| --- | --- |

- Maxwell distribution of energy scaled to kT is stated as:

| ϕME=1kT2Eexp-EkT | 3.35 |
| --- | --- |

- It integrates to unity: 
0∞x*e-xdx=1

- There is absorption that removes the low energy neutrons preferentially. This moves the average toward higher energy

## Fast and Thermal Reactor Spectra

Σ=N*σ→#cm=#cm3*cm2

Σ(E)=N(E)*σ→#cm=#cm3*cm2for E∈E, E+dE

n'''EdE→#cm3.eVeV=#cm3

ϕE=vE.n'''E→#cm2=cms#cm3.eV for E∈E, E+dE

# Energy-Averaged Reaction Rates

- For example, average microscopic cross section can be computed for a known atomic composition:

| 0∞σxEϕEdE=σxϕ | 3.40 |
| --- | --- |

- Similarly, the average speed of neutrons can be averaged over the population of neutrons:

| v=0∞vEn'''EdE / 0∞n'''EdE | 3.44 |
| --- | --- |

- Most importantly, the energy range can be broken into regions and cross section evaluated over individual regions:

| 0∞σxEϕEdE=σxϕ=TσxEϕEdE+IσxEϕEdE+FσxEϕEdE | 3.45 |
| --- | --- |

| Range Name | Value |
| --- | --- |
| Thermal | 0.001eV – 1.0 eV |
| Intermediate/Resonance | 1.0 eV – 0.1 MeV |
| Fast | 0.1 MeV – 10 MeV (or ∞) |

| σxΩ=ΩσxEϕEdEΩϕEdE, Ω=T, I, F | 3.48 |
| --- | --- |

- Advanced studies, break the energy range into finer groups (multi-group) but the above ranges are adequate to our basic purposes

## Fast Cross Section Averages

- Existence of resonance scattering in MeV region is responsible for rugged appearance

- A simplified approximation of the flux is the fast spectrum distribution thus:

## Resonance Cross Section Averages

| ϕ(E)=qξ ΣsEE | 3.26 |
| --- | --- |

## Thermal Cross Section Averages

- The most often used value of evaluation is E=0.0253 eV equivalent to 2200 m/s speed both correspond to T0=293.61 K

- The exact thermal spectrum is not well determined.

- The cross section is also function of the binding energy of the crystals (solid state chemistry!)

- Cross section depends on density, which is temperature dependent!

  - Σx=Nσ=ρN0Aσ

  - Thermal expansion causes cross section decrease!

- (Sidenote: this is how the sun as a fusion reactor regulates its thermal output!)

# Infinite Medium Multiplication

- Infinite medium multiplication can be approximated using macroscopic fission to absorption ratio.

- The macroscopic cross section is found by averaging over all nuclei constituting the reactor core material including fuel, structural and coolant/moderator materials.

- It should be noted too that averaging process over the entire energy range should be done.

| k∞=0∞νΣfEϕEdE0∞ΣaEϕEdE=νΣfΣa | 3.60 |
| --- | --- |

| k∞=νΣfΣa | 3.61 |
| --- | --- |

- It should be noted too that averaging process over the entire energy range should be done.

- The formula assumes that the flux is the same over the entire core and materials are well mixed

- This is not true and some spatial variations exist in the reactors as the fuel rods, moderators and coolant are separate spatially and thus spatial averaging should be considered

# Problems