# Ch.05.Reactor Kinetics

5.1	Introduction	1

5.2	Neutron Balance Equations	1

5.2.1	Infinite Medium Non-multiplying Systems	1

5.2.2	Infinite Medium Multiplying Systems	1

5.2.3	Finite Multiplying Systems	1

5.3	Multiplying Systems Behavior	1

5.4	Delayed Neutron Kinetics	1

5.4.1	Kinetics Equations	1

5.4.2	Reactivity Formulation	1

5.5	Step Reactivity Changes	1

5.5.1	Reactor Period	1

5.5.2	Prompt Jump Approximation	1

5.5.2.1	Rod Drop	2

5.5.2.2	Source Jerk	2

5.5.2.3	Rod Oscillator	2

5.6	Prologue to Reactor Dynamics	2

## Introduction

- Follow dynamics of neutron chain reaction or time behavior.

- We apply two simplifications:

  - Average all cross sections over the energy spectrum of the reactor

  - Apply spatial averaging to neutrons

- To study time kinetic in a simple way, average the other two dimensions and focus on time only now.

| Non multiplying (no fissionable material, ∞) ↓ Multiplying (i.e., with fissionable material, ∞) ↓ Multiplying and finite (i.e. with leakage) ↓ Multiplying + finite + delayed neutrons | 🔲 ↓ fissionable ↓ Fissionable + boundary ↓ Fissionable + boundary + delayed neutrons ↓ | nt=n0exp-tl∞ ↓ nt=n0exp-k∞-1l∞t ↓ nt=n0exp-k-1lt ↓ nt=n0exp-k-1lt ↓ nt=n0expt/T | k∞=νΣf/Σa l∞=1/Σav k=k∞*PNL l=l∞*PNL l=l+β/λ |
| --- | --- | --- | --- |

## Neutron Balance Equations

### Infinite Medium Non-Multiplying Systems

- In infinite non-multiplying medium, the neutron population changes due to either source or absorption:

ddtnt=St-Σavnt

- At time t=0, assume no source (S(t)=0) and initial population of n(0):

nt=n0 exp-t/l∞

- The mean “infinite”, lifetime of a neutron is given by:

l∞=0∞tntdt0∞ntdt=1Σav

- Let us have a constant source on top of absorption:

ddtnt=St-Σavnt=S0-Σavnt=S0-ntl∞→

nt=l∞S01-exp-tl∞

ddtnt=St+Constant*nt→nt=St=0nt=n0*exp⁡(-t/constant)St=S0nt=constant*S01-exp-t/constant

### Infinite Medium Multiplying Systems

- Add fissionable material on top of a source term and absorption:

ddtnt=St+νΣfvnt-Σavnt

- The infinite multiplication factor has been defined from before to be: k∞=νΣf/Σa

ddtnt=St+ΣavntνΣfΣa-1=St+nt1/ΣavνΣfΣa-1=St+ntl∞k∞-1=St+k∞-1 l∞nt

- So, the changes to the population are the source and whatever difference from unity the infinite multiplication factor gives out (positive or negative):

ddtnt=St+k∞-1 l∞nt

- If there is no source, the equation simplifies to:

ddtnt=k∞-1 l∞nt: k∞>1      supercriticalk∞=1                 criticalk∞<1           subcritical

nt=l∞S01-exp-k∞-1l∞t

nt=n0 exp-k∞-1l∞t

### Finite Multiplying Systems

- Now, add leakage term and model it to be proportional to the absorption term:

ddtnt=St+νΣfvnt-Σavnt-Γ Σavnt

- You may state that neutrons:

  - Born from:

    - External source St

    - Fission source

  - Die because of:

    - Absorption

    - Leakage from the system

- Thus, the probability of leakage compared to the total losses are:

PL=Γ1+Γ,    PNL=11+Γ

- As noted, the neutrons are removed by either leakage or absorption and thus:  PNL+PL=1

- Both are less than 1

- Multiply the above equation by the non-leakage term PNL:

PNLddtnt=PNLSt+PNLνΣfvnt-PNLΣavnt+Γ Σavnt

PNLddtnt=PNLSt+PNLνΣfvnt+PNLPNLΣavntddtnt=PNLSt+nt1/ΣavPNLνΣfΣa-1 =PNLSt+PNLPNLntl∞PNLk∞-1=PNLSt+ PNLPNLk∞-1PNLl∞nt

| ddtnt=St+ PNLk∞-1PNLl∞nt=St+ k-1lnt | 5.22 |
| --- | --- |

- Where, finite life and finite multiplication factor are: k=PNLk∞, l=PNLl∞, both smaller than their infinite counter parts.

## Multiplying Systems Behavior

- Assume that there is no source in Equation 5.22 which can be integrated to be:

| nt=n0exp k-1lt: k>1 supercritical, limt→∞n(t)→∞k=1 critical, limt→∞n(t)→n0k<1 subcritical, limt→∞n(t)→0 | 5.24 |
| --- | --- |

- Having a constant source (not varying in time St=S0), 5.22 can be integrated to 5.26:

| nt=l S0k-1exp k-1lt-1→∞ as t→∞ for k > 1 | 5.26 |
| --- | --- |

| nt=l S0k-11-exp -1-klt→n∞=l S0k-1 for k<1 | 5.27 |
| --- | --- |

| nt=l S0k-11+k-1lt+12k-12l2t2+…-1→n∞=S0t for k=1 | 5.28 |
| --- | --- |

- The time progress of neutrons’ population indicates linear increase in time

- Summary equation:

nt=l S0k-1exp k-1lt-1:k>1      supercritical, limt→∞nt=()exp+()t-1→∞k=1                                              critical, limt→∞nt→S0t =∞k<1                                              subcritical, limt→∞n(t)→l S01-k

- The time progress of neutrons’ population for the two cases of no source or constant source is shown in Figure 5.1

- Stable population is established for two cases only:

  - Critical multiplication (k=1) with no source present (S(t)=0)

  - Subcritical multiplication (k<1) with constant source present (S(t)=S0)

- It is very difficult to stabilize the population given the extremely fast time scale as the neutrons’ lifetime is very small in the range of 10-8-10-4s. Any small deviation from unity will either exponentially crash the population or increase it and both are not desired for stability.

f[k_, t_] := 1*Exp[(k - 1) t];

d = 1.5;

Plot[{f[1 - d, t], f[1, t], f[1 + d, t]}, {t, 0, 1},

GridLines -> Automatic, PlotRange -> {0, 3}]

d = 1.5;

g[k_, t_] := 2/(k - d)*(Exp[(k - d) t] - 1);

Plot[{g[1 - d, t], g[1, t], g[1 + d, t]}, {t, 0, 1},

GridLines -> Automatic, PlotRange -> {0, 3}]

## Delayed Neutron Kinetics

- 99% of neutrons are prompt and 1% is delayed. The ratio is called “delayed neutron fraction” β:

- The delayed neutrons are around 50 types but usually grouped in six characteristic delay times:

| β=i=16βi, 1λ=1βi=16βi*1λi | 5.31, 5.34 |
| --- | --- |

- Modify the lifetime parameter to account for the delay results in much longer lifetime

| ld=l+t1 /2 ln2=l+1/λ l=1-βl+βld=1-βl+β(l+1λ)=l+β/λ | 5.35, 5.36 |
| --- | --- |

- This modification is significant since: βλ≫l

### Kinetics Equations

- Include the effects of the delayed neutrons by dividing the fission term into:

  - Prompt fission with slightly reduced factor of (1-β)

  - Delayed neutrons are kind of decay terms of precursor with concentration, C, and half-life ti12

    - Source: fission is the source (with βi fractions)

    - Sink: decay of the concentration product similar to any exponential decay

| ddtnt=St+1-βνΣfvnt+iλiCit-Σavnt-Γ Σavnt | 5.37 |
| --- | --- |

- We obtain the following modified differential equation:

| ddtnt=St+1l1-βk-1 nt+iλiCit | 5.40 |
| --- | --- |

- The time variation of the pre-cursor concentration changes is given by:

| ddtCit=βiklnt-λiCi, i=1, 2,…6 | 5.41 |
| --- | --- |

- To have stable solution or zero changes in both neutron population, n, and concentration, C:

  - Assume constant source

  - Zero change in population

  - Zero change in concentration of precursors

| 0=S0+1l1-βk-1 n+iλiCi |  |  | 5.42 |
| --- | --- | --- | --- |
| 0=βikln-λiCi, i=1,2,…, 6 | 5.43 |

- We need to divide the situation into having constant source or not

- If there is a constant source, then 5.42 equation simplifies!

| 0=S0+1l1-βk-1 n+iλiCi=S0+1l1-βk-1 n+klniβi=S0+1l1-βk-1 n+βlkn=S0+k-1l n →n=lS01-k | 5.44 |
| --- | --- |

- To have the neutron population steady and positive, there are two cases:

  - For a constant source S0≠0: k must be less than 1 (sub critical)

  - For no sourceS0=0: k must be one or critical to have n non-zero.

- In both cases, the delayed neutrons have no effect on the final stability value of the solution. The time evolution toward stability is different from the stability value.

### Reactivity Formulation

- We want to recast the differential equations in terms of two modified parameters: reactivity ρ (small rho), and prompt lifetime Λ (capital lambda):

| ρ≡k-1k k∞-1k∞=νΣf/Σa-1νΣf/Σa=νΣf-ΣaνΣf=netfission | ᵨ5.45 |
| --- | --- |

- The reactivity can be thought of as the ratio of the net neutron production rate to the neutron production rate due to fission alone

- The prompt generation time is defined as:

| Λ≡lk | 5.45 |
| --- | --- |

- With the above two definitions the differential equations can be re-expressed in reactivity terms:

| ddtnt=St+kl*1kk-1+βk nt+iλiCit ddtnt=St+1Λρ+β nt+iλiCit ddtCit=βinΛ-λiCi, i=1,2,…6 | 5.47 5.48 |
| --- | --- |

- Under steady state condition, there are more nuclei emitting neutrons (Ci(t)) than neutrons (n(t)) which simplifies the steady state solution to the following constant with two cases (constant source with rho non-zero) or exponential

n=ΛS0ρ

## Step Reactivity Changes

- What happens when a step change in reactivity is applied to an initially critical reactor that has been operating at steady state? Perturb, and see?

- The perturbation is small: a fraction of delayed neutron fraction.

- The two curves are the solution to the differential equations 5.47 and 5.48.

  - Nothing dramatic: reasonably slow multiplication of population in 1-2 minutes given the step size increase in reactivity

  - (This timeframe allows some electromechanical control by inserting control rods.)

- Early (at the start) very fast increase (or decrease) followed by slow growth (or decline)

  - The fast increase (or decrease) is due to the prompt neutrons

  - The slow behavior is due to the delayed neutrons

- The behavior can be approximated with a general formula:

nt=constant .exptT=constant .expωt

- T is called the period, or the reactor characteristic time if you like,

- T is time required for the reactor power to increase or decrease by a factor of e and is arguably the most important quantity derivable from the kinetics equations.

### Reactor Period

- In the following, we try to find the reactor period based on the basic differential equations governing neutrons including delayed fraction:

| # | Step | Equation |  |
| --- | --- | --- | --- |
| 1 | Assume neutron population and pre-cursors to be in exponential forms A, Biand ω are assumed constants (not the solution only form!) | --- ddtnt=St+1Λρ+β nt+iλiCit ddtCit=βinΛ-λiCi, i=1,2,…6 | 5.47 5.48 |
| 2 | 7 (1+6) differential equations 5.47 and 5.48 Have the following solutions | ωA=ρ-βΛA+iλiBi ωBi=βiΛA+λiBi | 5.49 5.50 |
| 3 | Using the above equations and the beta cross section (β=i=16βi) | ωA=ρ-βΛA+AΛiλiβiω+λi ω=ρ-β*1Λ+1Λiβi λiω+λi=ρΛ-1Λiβi ω+λiω+λi+1Λiβi λiω+λi=ρΛ+1Λi-βi ω+λi+βi λiω+λi=ρΛ+1Λi-βi ωω+λi ω=ρΛ-1Λiβi ω+λiω | 5.51 5.52 |
| 4 | Rearrange the last equation to find reactivity: “inhour” equation Reactivity is function of a single prompt term and 6 delayed terms | ρ=Λ+iβi ω+λiω If there are no delayed neutrons (i.e., βi=0): ρ=Λω→ρT=lk→T=lkρ=lk(k-1)/k=lk-1 | 5.53 |
| 5 | The plot of RHS of the inhour equation gives the following figure (ρ=fω) There are 7 roots for any selected ρ value including ρ=0. Since, the reactivity is desired to be zero, there are only specific values of ω that satisfies this criterion. |  |  |
|  | Plotting a simple example | y(x)=1-2x+4-2x+6x y=0→x=0, -3-5,-3+5={0, -5.2, -0.8} |  |
| 6 | We must have the neutron population in the following form ω1>ω2>ω3>ω4>ω5>ω6>ω7 | nt=i=17Aiexp(ωit) |  |
| 7 | One term dominates the longtime thus others are dropped: | nt≈A1exp(ω1t)=A1exp(t/T) | 5.55 |
| 8 | When the reactivity is small, so is the largest value of omega,ω1. This enables us to remove multiple terms and keep one in reactivity: asymptotic solution | ρ=Λ+iβi ω+λiω→Λ+iβi λiω1=Λ+βλω1 |  |
|  | “Thus, for small reactivities—positive or negative—the reactor period is governed almost completely by the delayed neutron properties β and λ.” | Since Λ≪ β/λ: ρ=βλω1=βλ1T → T≈βλρ So, for small reactivity, ρ≤± β, the reactor period is governed by a time constant totally controlled by delayed neutrons. The period is in order of minutes which can be controlled by electromechanical means. |  |
|  | Above prompt critical, ρ>β, the period is very small. | T≈Λρ-β So, the reactor is extremely sensitive and potentially explosive If you jerk the reactivity by large amount, ρ>β , the output multiplies in a period comparable to milliseconds! Prompt criticality is not forgiving, and output of the reactor can multiply to explosive levels. |  |
|  | Reactivity measured in $. | $=ρβ |  |

### Prompt Jump Approximation

#### Rod Drop

- Rod drop is a reactivity perturbation method

- Masterson: A study of the transient response of the reactor to a rapid insertion of high negative reactivity

  - dropping the reactor control rods when the reactor is in a critical state

  - immediately after control rod insertion ρ1=0 which allows estimating reactivity

- Lewis: from steady state  Immediately following the rod insertion

  - In the immediate time after the rod drop and much less than decay of precursor:

- A solution to this equation can be proven to be, n0=n0:

- In some time above the prompt regeneration time and less the decay :

- The reactivity drop in units of beta or $, can be found:

- This is an experimental method to measure the reactor’s reactivity if the perturbation is within a small fraction of beta (i.e. small even compared to delayed fraction):

#### Source Jerk

- The source jerk is performed on a subcritical reactor,ρ=-ρ, containing a neutron source So.

- Removing the source from a subcritical system with reactivity ρ=-ρ,  (the source jerk experiment) results in a measurable drop in the neutron population from no to n1.

#### Rod Oscillator

- Another experimental method to measure the reactor’s reactivity is to oscillate the control rod:

- After early transients, the population is stated:

- plotting n/n0 versus frequency determines ρ0/Λ, the ratio of reactivity to prompt neutron generation time

## Prologue to Reactor Dynamics

- The analysis so far does not include thermal feed back effects. It is basic and is called “zero-power kinetics.”

- As temperature changes:

  - Density changes

  - Cross sections change

  - Doppler broadening increases

  - Maxwell distribution of neutrons’ energy shifts (harder spectrum for hotter values)

- Temperature feedback could be positive or negative (we desire negative feedback)

- The following figure (log y scale) shows that negative feedback is desired to control overpower.

- Sun’s core as a fusion reactor has negative feedback:

  - As temperature rises, the core expands and rate of fusion decreases

  - As temperature cools, the gravity densifies the core and fusion rate increases

# Problems

l∞=1Σav=12.2×105cm s-1×0.273×10-3 cm-1=0.01665 s=17. mS

The average velocity change with KE:

KE=12m v2→v∝E

v2=v1*E2E1=2.2×105cm s-1*106eV0.0253 eV=1.38×109cm/s

l∞=1Σav=11.38×109cm s-1×0.738×10-3 cm-1=9.82*10-7s=0.982 μS

- Finding the desired period of the reactor:

nt=n0exp-k-1lt=n0exp-t/T

ntn0=3=exp-tT→Tt=1ln3→T=10sln3=9.1 S

- Finding the needed reactivity to achieve the above period:

nt=n0exp-k-1lt=n0exp-ρΛt

lnntn0=-k-1lt

ln3=-k-10.002s10s

k=1+0.002*ln310=1.000219722=1.000220

ρ=k-1k=0.000219674=0.000220

- Period is found using the generic definition of reactor period:

nt=n0exp-k-1lt=n0exp-t/T

ntn0=103=exp-tT→Tt=1ln3→T=tlnnn0=2hrln103=0.2895 hr=17.4 m

- Since the reactivity is done slowly (cents), we could use the approximate formula:

T≈βλ ρ→ρβ=1λ T=1λ 17.4*60 s=10.0766 s-1*17.4*60 s=0.0125046$=1.25 ¢

Let us assume that we are dealing with U-235 and obtain the beta and lambda from Table 5.1:

1λ=1β*βiλi

λi=log2T12,i

λ=0.766 s-1

We had computed decay constant for U-235:

λ=0.766 s-1

T≈βλ ρ=β ρ1λ=β+0.01β*10.766 s-1=1305.48Sm60S=21.8 m

T≈βλ ρ=β ρ1λ=β-0.01β*10.766 s-1=-1305.48Sm60S=-21.8 m

ρβ=n0n1-1=8060-1=0.33$

dNtdt=A0-λNt→λNt=A01-e-λt as in Chapter 1.5 assuming N0=0