# 07.Neutron Distributions in Reactors

7	Neutron Distributions in Reactors	2

7.1	Introduction	2

7.2	The Time-Independent Diffusion Equation	2

7.3	Uniform Reactors	3

7.3.1	Finite Cylindrical Core	3

7.3.2	Reactor Power	5

7.4	Neutron Leakage	7

7.4.1	Two Group Approximation	7

7.4.2	Migration Length	9

7.4.3	Leakage and Design	10

7.5	Reflected Reactors	11

7.5.1	Axial Reflector Example	11

7.5.2	Reflector Savings and Flux Flattening	13

7.6	Control Poisons	14

7.6.1	Reactivity Worth	14

7.6.2	Partially Inserted Control Rod	15

7.6.3	Control Rod Bank Insertion	16

7.7	Problems	17

# Neutron Distributions in Reactors

## Introduction

- In chapter 6, several geometries were selected based on their “mathematical” simplicity including:

  - Semi-infinite media

  - Limited slap of fissionable material

  - Spherical geometry of fissionable material

- In this chapter, we focus on a more realistic cylindrical geometry

  - Fission is included

  - Energy is first considered as a single group then taken to be two groups (fast and thermal)

- Toward the end of this chapter, several features will be included:

  - Reflective boundary

  - A single control rod

  - Control rods bank

## The Time-Independent Diffusion Equation

- We start with steady state (in t), diffusion equation

  - with no source and

  - with fission coefficient

| ∇.D∇ϕr+νΣfϕr-Σaϕr=0 | ‎7.1 |
| --- | --- |

- Sensible design strategy is to modify reactor’s geometry, and materials composition to find optimal reactor behavior

- To find a mathematical solution to this PDE, one might go for “designing” proper coefficients

  - Let us assume that neutron production per fission, ν, can be controlled

  - The value ν0 is the value that makes the reactor exactly critical

| ∇.D∇ϕ+(ν0/ν)νΣfϕr-Σaϕr=0 | ‎7.2 |
| --- | --- |

- The deviation of neutrons from ν0, is the multiplication thus

  - The multiplication factor is: k=ν/ν0

| ∇.D∇ϕ+1kνΣfϕr-Σaϕr=0 | ‎7.3 |
| --- | --- |

- The above equation is an eigen value/eigen function problem for which many solutions exist:

| ∇2ϕr+constant ϕr=0 0<ϕr<∞for r∈Vthereactorbody |  |
| --- | --- |

- Since this problem represent a physical system, the following physical condition apply:

  - The flux cannot be negative within the volume of the reactor

  - The flux must be finite (< ∞)

## Uniform Reactors

- Take the reactor cross section coefficients to be uniform:

  - Thus, the diffusion coefficient is constant

  - The previous definitions of the infinite multiplication factor and the diffusion length:

  - k∞=νΣf/Σa,	L2=D/Σa , 1D(ν0/ν)νΣf-Σa=ΣaD(ν/ν0)-1  νΣfΣa-1=1L2k-1k∞-1=k∞/k-1L2

- The diffusion equation can be expressed in the following form:

| ∇2ϕ+k∞/k-1L2ϕ=0 | ‎7.4 |
| --- | --- |

- An insight can be gained by reformatting the spatially dependent factors in one side and the spatially independent factors in the other:

| -∇2ϕrϕr=k∞/k-1L2=B2 | ‎7.5 |
| --- | --- |

- Express the multiplication using the newly defined constant:

| k=k∞1+L2B2 | ‎7.6 |
| --- | --- |

- We have the PDE expressed as a Helmholtz equation, a named form of PDE,

  - The constant B is named as the geometric buckling, or buckling

| ∇2ϕ+B2ϕ=0 | ‎7.7 |
| --- | --- |

- Since this represents a physical system, the solution must also satisfy:

  - 0<ϕr<∞for r∈Vthe reactor body

- The multiplication ratios represent the non-leakage coefficient as follows:

  - k=PNLk∞

  - The non-leakage coefficient is written in terms of the diffusion length and buckling:

| PNL=11+L2B2 | ‎7.8 |
| --- | --- |

- The last three equations are valid for any reactor’s shape.

### Finite Cylindrical Core

- Let us have a uniform cylindrical reactor, with an extrapolated radius R and extrapolated height H

- The Laplacian operator in cylindrical coordinates can be written in terms of the radial (polar radial) and axial coordinates r and z:

| 1r∂∂r rddr ϕ+∂2∂z2ϕ+B2ϕ=0 | ‎7.9 |
| --- | --- |

- The following physical condition applies:

| 0<ϕr,z<∞, 0≤r≤R, -H/2≤z≤ H/2 | ‎7.10 |
| --- | --- |

- Separation of variables (ψ/psi and χ/chi):

| ϕr,z=ψrχz | ‎7.11 |
| --- | --- |

- Inserting it in the equation ‎7.7, and dividing the equation by the function ϕ=ψχ:

| 1ψ1r∂∂r rddr ψ+1χ∂2∂z2χ+B2=0 | ‎7.12 |
| --- | --- |

- In the above equation, the first term is function of the radius only, the second on the axial variable z only and the last term is a constant.

- Thus, the first two terms must be also constants: -Br2,  and -BZ2, or:

| Br2+BZ2=B2 | ‎7.13 |
| --- | --- |

- Now the PDE is written as two ODE, (i.e., of a single variable each):

| 1r∂∂r rddr ψr+Br2ψr=0, 0≤r≤R | ‎7.14 |
| --- | --- |

| ∂2∂z2χz+Bz2χz=0, -H/2≤z≤ H/2 | ‎7.15 |
| --- | --- |

- The second equation, Eq ‎7.15,  has the known general solutions of sine and cosine:

| χz=C1sinBzz+C2cosBzz, -H/2≤z≤ H/2 | ‎7.16 |
| --- | --- |

- A valid boundary condition is to have the flux zero at the boundary z=±H/2

- Symmetry of the physical reactor in z exclude the sine function sin-z=-sinz thus C1=0

- The boundary condition:

  - cos±Bz H/2=0→  ±Bz H/2=π/2, 3π/2 , …

- Since the cosine is positive only within the first loop, only the first root is accepted (i.e., π/2)

| Bz=π/H→→χz=C2cosπ z/H | ‎7.17 |
| --- | --- |

- The ODE equation, Eq ‎7.14 has an analytic solution called Bessel functions of the first and the second types (both are order zero).

| ψr=C1J0Brr+C2Y0Brr, 0≤r≤R | ‎7.18 |
| --- | --- |

- Bessel function of the second type is not finite near zero (limr→0Y0r→-∞) thus we set C2=0

- Bessel function of the first type is positive only within the first loop.

- We can set the boundary condition to be zero at the first zero of the Bessel function (@ r=2.405)

  - J0Br R=0

| Br=2.405/R→ψr=C1J02.405r/R | ‎7.19 |
| --- | --- |

- Both radial and axial constants are known and thus, it can be stated:

| B2=2.405/R2+π/H2 | ‎7.20 |
| --- | --- |

- Now the general solution is known, up to a constant:

| ϕr,z=C J02.405 r/Rcosπ z/H | ‎7.21 |
| --- | --- |

- The unknown constant is still to be determined: C=C2 C1

### Reactor Power

- We can link the unknown constant, C, to the current flux of the reactor based on the average power!

  - The flux is proportional to the fission rate

  - The fission rate is directly proportional to the power through the average energy per fission:

  - The power at any point is the fission coefficient times the flux (fissions) at the point

  - Integration gives the total number of fissions

  - The gamma converts the total fissions to power:

| P=γ∫Σfϕ dV | ‎7.22 |
| --- | --- |

- Write the integration into cylindrical coordinates with limits:

  - 2π  is the integral of polar angle over a full revolution.

| P=γ2π0R-H/2H/2Σfϕ dz rdr | ‎7.23 |
| --- | --- |

- Separate the integration into two integrals:

  - Define the volume of the cylinder: V=πR2H

| P=CΣfγ2π0RJ02.405 r/Rrdr-H/2H/2cosπ z/Hdz | ‎7.24 |
| --- | --- |

| P=CΣfγV2R20RJ02.405 r/Rrdr1H-H/2H/2cosπ z/Hdz | ‎7.25 |
| --- | --- |

- Let us do the following change of variables:

  - ξ=2.405 r/R,

  - ζ=πz/H

| 2R20RJ02.405 r/Rrdr=22.405 202.405J0ξξdξ=22.405J12.405 | ‎7.26 |
| --- | --- |

| 1H-H/2H/2cosπ z/Hdz=1π-π/2π/2cosζdζ=2π | ‎7.27 |
| --- | --- |

| P=CΣfγV22.405J12.4052π=CΣfγV*0.275 | ‎7.28 |
| --- | --- |

| C=3.63PΣfγV | ‎7.29 |
| --- | --- |

| ϕr,z=3.63PγΣfVJ02.405rRcosπzH | ‎7.30 |
| --- | --- |

- The Bessel and cosine function look similar, but they are different!

## Neutron Leakage

- The diffusion length measures the distance that neutrons travel between birth and death

  - If diffusion coefficient and cross sections used are averaged over only the thermal neutron energy spectrum, gives significant errors.

  - The diffusion length is averaged over fast and thermal separately skipping the epithermal due to its small relative population (and time spent by neutrons as epithermal).

### Two Group Approximation

- Define

  - The fast neutrons’ flux ϕ1

  - The thermal neutrons’ flux ϕ2

- The fast neutrons are generated mainly from thermal neutrons undergoing fission: ϕ2

  - Thermal neutrons being absorbed: Σa ϕ2

    - in units of: Σa ϕ2 /cm3/s

  - Only count those absorbed in the fuel: fΣa ϕ2

  - f: is the fraction of absorption by the fuel and not moderator or structures.

  - the number of fast neutrons produced from thermal fission: ηT

  - 1/k to account for variations of the neutrons in case of deviation from criticality

    - In similar basis to equation 7.3

  - ϵ  extends the number in case of fast fission neutrons

- The diffusion equation for fast neutrons:

  - The first term is the leakage term (boundary accounting)

  - The second term is the removal cross section, Σr ,

    - due to slowing down of fast into thermal

    - In this fast flux, this is a sink (loss) term

    - In the thermal flux, this will be a source (gain) term

| ∇.D1∇ϕ1+Σrϕ1=1kϵηTfΣa ϕ2 | ‎7.31 |
| --- | --- |

- The diffusion coefficient for fast neutrons is:

| D1=1/3Σtr1 | ‎7.32 |
| --- | --- |

- The diffusion equation for thermal neutrons:

  - The first term is the leakage term (boundary accounting)

  - The second term is the absorption of the thermal neutrons: Σa

  - The thermal neutrons are created by removal and slowing fast into thermal: Σr

- the resonance escape probability p to account for those lost to capture in the fuel resonance

| ∇.D2∇ ϕ2+Σa ϕ2=pΣr ϕ1 | ‎7.33 |
| --- | --- |

- The thermal diffusion coefficient is:

| D2=1/3Σtr2 | ‎7.34 |
| --- | --- |

- Divide the equations:

  - Fast diffusion ϕ1eqn by Σr

  - thermal diffusion ϕ2eqn by Σa

- Define two diffusion lengths:

  - Fast diffusion length L1

    - These fast neutrons are “removed” after wondering around

    - They will be thermal neutrons by removal, not an execution!

  - Thermal diffusion length L2

    - These thermal neutrons are “absorbed” after wondering around

    - They will be fast neutrons by absorption/fission, this is an execution!

| L1=D1/Σr | ‎7.35 |
| --- | --- |

| L2=D2/Σa | ‎7.36 |
| --- | --- |

- Now, the fast and the thermal diffusion equations can be expressed in terms of diffusion lengths:

| -L12∇2ϕ1+ϕ1=1kϵηTfΣaΣr ϕ2 | ‎7.37 |
| --- | --- |

| -L22∇2ϕ2+ϕ2=pΣrΣa ϕ1 | ‎7.38 |
| --- | --- |

- Based on our previous similar derivations in section 7.1-7.3, we assumed:

  - Zero flux at the boundary

  - The PDE can be written as simple ODE with buckling factors:

| ∇2ϕ1+B2ϕ1=0 | ‎7.39 |
| --- | --- |

| ∇2ϕ2+B2ϕ2=0 | ‎7.40 |
| --- | --- |

- These equations of flux allow one to write them as constants (with diffusion lengths and buckling factors):

| ϕ1=11+L12B21kϵηTfΣaΣrϕ2 | ‎7.41 |
| --- | --- |

| ϕ2=11+L22B2pΣrΣa ϕ1 | ‎7.42 |
| --- | --- |

- The above two equations can be combined, say by substituting 7.42 into 7.41

  - As per chapter 4, four factors formula: k∞=ϵ p f ηT

| ϕ1=11+L12B21kϵηTfΣaΣr*11+L22B2pΣrΣa ϕ1 →k=11+L12B211+L22B2pϵηTf, k=11+L12B211+L22B2k∞ | ‎7.43 |
| --- | --- |

- As per, k=PNLk∞, the non-leakage probability is then given by:

| PNL=11+L12B211+L22B2 | ‎7.44 |
| --- | --- |

- Thermal diffusion length, D2, definition is adequate (D2=1/3Σtr2)

- To estimate the fast diffusion length an approximate formula, known as the “Fermi age” is used:

  - It is the most frequently employed approximation for L12.

  - “Basically, the Fermi age is a measure of how far a neutron travels in graphite, similar to the "diffusion length". It is called "age" because the neutron lifetime is proportional to how long it takes to slow down in a moderator.”

  - Σs: scattering cross section

  - D: diffusion coefficient

  - The limits are: E1= 2.0 MeV to E2= 0.0253 eV

  - ξ: lethargy or ln(E0/E)

| τ=E2E1DEξ ΣsEEdE | ‎7.45 |
| --- | --- |

| L12=τ | ‎7.46 |
| --- | --- |

- a strong moderator results in a small diffusion length for fast neutrons

- we approximate the diffusion coefficient and scattering cross section as energy independent:

| L12≈D1ξΣslnE1/E2 | ‎7.47 |
| --- | --- |

- Based on this result we can deduce that removal coefficient cross section is approximately given by:

Σr=ξΣs/lnE1/E2

- Recall that the number of collisions is:

n≈1/ξ lnE1/E2→Σr≈Σs/n

- So, the removal cross section is scattering cross section  divided by number of collision needed to thermalize fast neutrons

L1≈nD1/Σs

- The smaller number of collisions needed the shorter the fast diffusion length.

### Migration Length

- The two energy groups analysis done above (fast and thermal) is done but is not as simple as a one group analysis.

- We could keep the improvements of from this two groups treatment by keeping the two diffusion lengths and defining a new length, we call migration length M.

- Carry out the multiplication in 7.44

  - B2 is a small factor for large reactors:B2=2.405/R2+π/H2

  - This allows us to ignore the B4term:

| PNL=11+(L12+L22)B2+L12L22B4 | ‎7.48 |
| --- | --- |

- Define the summation of the two diffusion length squares as the migration length squared M2

B2 is a small factor which allow us to ignore the B4term:

| PNL=11+M2B2 | ‎7.49 |
| --- | --- |

| M2=L12+L22 | ‎7.50 |
| --- | --- |

| M=L12+L22 | ‎7.51 |
| --- | --- |

- Table 7.1 lists the values of L1 (=τ ), L2 , and M for the three most common moderators, and their representative power reactors using each of them

- The new analysis improves upon thermal diffusion length L2 using M.

- The largest correction is for that of light water reactors

  - Hydrogen relatively large absorption causes it to have low L1 making L2 significant

    - L2=D2/Σa

    - L1≈nD1/Σs

  - Hydrogen, also, relatively low scattering causes L2 be large

- For fast reactors, fast diffusion L2 and migration lengths are essentially identical

  - Example: a sodium-cooled fast reactor (SFR) is M = 19.2 cm

  - A gas-cooled fast reactor (GCFR), M = 25.5 cm.

### Leakage and Design

- The relationship between neutron leakage and power reactor design can be investigated:

- We start by using, k=PNLk∞, to have:

| k=11+M2B2k∞ | ‎7.52 |
| --- | --- |

- If we take: H=2R , the buckling becomes

  - B2=2.405/R2+π/H2=33/H2

- We have:

| k=11+33.0(M/H)2k∞ | ‎7.53 |
| --- | --- |

- Thus, the primary determinant of neutron leakage is H/M, the characteristic dimension of the reactor, measured in migration lengths.

  - From Table 8.2: H/M 40-55 for water cooled reactors

- as the volume and therefore H/M increases, the non-leakage probability becomes closer to one

  - that is, the leakage probability decreases

## Reflected Reactors

- Reflection, of light say, is to turn around photons from the reflector back to the source side. This process is not perfect but can be made very high fraction.

- Even x-rays (electromagnetic radiation) cannot be “reflected” in the same way, and we do not have good x-ray optics, for example (there are no x-ray mirrors or lenses)

- Particles cannot be reflected in the same manner as light. However, if you scatter back some particles, neutrons in our case, the loss of neutrons is reduced.

- “Some fraction of the neutrons that escape the core will make a sufficient number of scattering collisions in the diffusing reflector material to turn them around such that they reenter the core—that is, they are reflected back into to it”

- A reflector thus reduces the fraction of neutrons leaking from the reactor. Figure 7.4 shows schematic diagrams of cores with axial and radial reflectors.

- Reflectors have their largest impacts on smaller cores, where the leakage probability is significant. As we shall illustrate, a reflector’s importance diminishes as the size of a reactor—measured by H/M becomes larger.

- Masterson: Reflected reactors have three key advantages:

  - 1) First, a reflected reactor does not have to be as large as an un-reflected reactor to generate the same amount of power.  (Less expensive to build, and even possibly to operate)

  - 2) Secondly, the flux shape from the middle of the core to the edge is flatter. (More uniform fuel consumption and more uniform heat generation).

  - 3) Finally, a reflected reactor requires less fuel to achieve a critical mass than its non-reflected counterpart. Modeling the reactor is more challenging.

- Equation (7.9, 1r∂∂r rddr ϕ+∂2∂z2ϕ+B2ϕ=0) for the neutron distribution within a uniform core remains valid for reflected reactors. Three cases results:

  - both axial and radial reflectors are employed:

    - The separation of variables is not applicable, & more advanced mathematical methods are required.

  - Only a radial reflector or only axial reflectors

    - The separation of variables is still valid applicable,

    - The boundary conditions are different

      - Axial reflector: the height H of the critical core to decrease

      - Radial reflector: the core radius R to decrease

- If a reflector is added and the core dimensions are kept the same,

  - the multiplication will increase,

  - this must be compensated by decreasing k∞, for example, by:

    - reducing the fuel enrichment

    - or adding a neutron absorber to the core.

- We treat only the axial reflector here; the effects would be analogous for a radial reflector.

### Axial Reflector Example

- The reflecting material has no fissionable material thus PDE describing neutrons in the reflecting material has no source term:

| ∇2ϕ-1M2ϕ=0 | ‎7.54 |
| --- | --- |

- The diffusion length now is M, the migration length of the reflector material.

- In cylindrical geometry the Laplacian differential operator ∇2 in Eq. (7.54) takes the cylindrical form:

| 1r∂∂r rddr ϕ+∂2∂z2ϕ-1M2ϕ=0 | ‎7.55 |
| --- | --- |

- We again separate radial and axial variables (psi, zeta)

| ϕr,z=ψrζz | ‎7.56 |
| --- | --- |

- Dividing the equation by the flux permits writing it as follows:

| 1ψ1r∂∂r rddr ψ+1ζ∂2∂z2ζ-1M2=0 | ‎7.57 |
| --- | --- |

- The same logic applies allowing us to propose two constants: to have:

| α2=Br2+1M2 | ‎7.58 |
| --- | --- |

- The axial ODE now is given by:

| d2dz2ζz-α2ζz=0 | ‎7.59 |
| --- | --- |

- It must have a combination of exponential solutions:

| ζz=C"1expαz+C"2exp-αz | ‎7.60 |
| --- | --- |

- The exponential solutions can also be expressed in hyperbolic sine and cosine functions:

| ζz=C'''1sinhαz+C'''2coshαz | ‎7.61 |
| --- | --- |

- The boundary condition demands zero flux at the external edge of the reflector

- We next add a reflector of height a to the top and bottom of the core.

- Adding the reflector reduces the height of the critical reactor from H to a yet to be determined value of H'

- The boundary condition at the top of the reflector is: ζH'/2+a=0

- This boundary condition removes one of the arbitrary coefficients from Eq. (7.61).

  - If the boundary condition is applied, C2 coefficient becomes negative multiple of C1

  - C2=-ⅇ2(a+H2)αC1

  - This gives the following solution, with one less arbitrary constant,

| ζz=C'sinhαH'/2+a-z | ‎7.62 |
| --- | --- |

- The solution in the core once again takes the form of Eq. (7.16) with C1=0, because the solution still must be symmetric about the core midplane. However, the axial buckling, denoted here as Bz', is yet to be determined since we no longer employ χ±H/2=0, the axial boundary condition for a bare reactor. Thus, in the core we have:

| χz=C2cosBz' z | ‎7.63 |
| --- | --- |

- The interface condition dictates continuity of flux across the core–reflector interface:

| χH'/2=ζH'/2 | ‎7.64 |
| --- | --- |

- The current must also be continuous, (D is the reflector diffusion coefficient):

| DddzχzH'/2=DddzζzH'/2 | ‎7.65 |
| --- | --- |

- Inserting Eqs. (7.62), and (7.63) into Eqs. (7.64) and (7.65) we obtain the following two equations:

| C2cosBz' H'/2=C'sinhα a | ‎7.66 |
| --- | --- |

| Bz' DC2sinBz' H'/2=α D C'coshα a | ‎7.67 |
| --- | --- |

- Taking the ratio of these equations then yields:

| Bz' DtanBz' H'/2=α D cothαa | ‎7.68 |
| --- | --- |

- For thick reflectors (i.e., ones which are several diffusion lengths thick) the fraction of neutrons that escapes from the outer reflector surface becomes negligible. We may then approximate the reflector as infinite, taking a→∞. Because coth∞=1, Eq. (7.68) reduces to:

| Bz' tanBz' H'/2=α D/D | ‎7.69 |
| --- | --- |

| H'=2Bz'arctanαDBz'D | ‎7.70 |
| --- | --- |

- Note that once the radial dimension of the reactor and the reflector material properties have been specified, α, given by Eq. (7.58, α2=Br2+1/M2), and D are specified. For simplicity we assume that D is fixed.

- Then of the two remaining quantities, Bz' and H', we may fix one and determine the other.

### Reflector Savings and Flux Flattening

- We proceed by applying the foregoing formulas to two situations  H'=2Bz'arctanαDBz'D.

- In the first we specify, Bz'→Bz=π/H

- This is equivalent to stating that the core composition is the same as it would be for a bare core of length H.

- Replacing the buckling in Eq. (7.70) then gives

| H'=2HπarctanαHDπD | ‎7.71 |
| --- | --- |

- which may be shown to be less than H. The reduction in the half core height of the critical reactor is defined as the axial reflector savings: δz=1/2 H-H'.

- For a thick reflector, it is approximately: δz=MD/D. Figure 7.5 provides a comparison of the flux distributions for a bare and a reflected reactor in which the core composition is held constant, and the axial dimension reduced to maintain criticality.

- The treatment of the radial reflector is analogous, with the complication that Bessel functions are involved. Applying the thick reflector approximation to the radial reflector savings defined by: δr=R-R', we would obtain  δr=MD/D, again for a thick reflector.

- Masterson: “The reflector savings is a measure of how much the size of a reactor core can be reduced in each dimension if it is surrounded by a neutron reflecting material. The fraction of the neutrons that are reflected back into the core by the reflecting material is called the reflection coefficient or the albedo.”

- The second situation arises from specifying that the core height remains constant by taking H'=H. Solving Eq. (7.70) then yields a value Bz'<π/H. Consequently, replacing Bzwith the smaller value Bz' in Eq. (7.13) reduces the overall buckling in Eq. (7.6). Since the denominator decreases with the addition of the reflector, the value of k∞ must also be decreased—most likely by reducing the fuel enrichment in order for the multiplication to remain unchanged. Figure 7.6 compares the normalized flux distributions for bare and reflected reactors in which the height is held constant, and the composition changed to maintain criticality.

- To recapitulate, adding a reflector to a reactor allows either

  - (1) the volume of the reactor

  - (2) or the value of k∞ to be reduced,

  - (3) or some combination of the two.

- Figure 7.6 also illustrates a third effect: Adding a reflector flattens the flux distribution, thus lowering the ratio of peak to average flux.

- These effects, however, become less significant as the size of the reactor—measured in migration lengths—increases.

- Clearly, the reflector savings amounts to a smaller fraction of the core dimension as its size becomes larger. Likewise, if the core dimensions are held constant, adding a reflector has a smaller effect on the multiplication of a large reactor than on a small one. Table 7.2 compares the maximum and minimum flux for two reflected reactors with height to migration length ratios of  H/M=10 and H/M=50 to a bare reactor, with each of the three average fluxes normalized to one. The numbers clearly indicate the reflector’s diminishing effect on flux flattening as the size of the reactor increases.

## Control Poisons

- Control poisons take three forms:

  - control rods

  - soluble poisons dissolved in liquid coolants

  - burnable poisons permanently embedded in the fuel or other core components

- Control poisons/rods

  - control the value of k:

    - to startup,

    - to shutdown

    - to modify power level

  - Compensating for (still through k):

    - fuel depletion,

    - fission product buildup,

    - temperature changes,

- Simplifications:

  - uniform un-reflected reactor to which we add a control poison

  - No effects on fission coefficient

  - Negligible effects on diffusion coefficient

  - Primary effect is an increase in the absorption cross section

Σa→Σa+δΣa

  - This absorption effect is assumed uniform across the core with no spatial variations

    - boron absorber that is added to the coolant PWR making it uniform

    - Control rods effect is localized

    - Burnable poisons typically are distributed to optimize the neutron distribution (non-uniform but out of scope)

### Reactivity Worth

- We derive a general expression for the reactivity decrease resulting from the additional neutron absorption by the poison

  - “The worth of the absorber”

  - k, ϕ→k', ϕ'

- We start by re-expressing PDE equation of ‎7.3  (∇.D∇ϕ+1kνΣfϕr-Σaϕr=0) in terms of the new reduced flux and multiplication:

| D∇2ϕ'+1k'νΣfϕ'-Σa-δΣaϕ'=0 | ‎7.72 |
| --- | --- |

- We multiply by non-poisoned flux ϕ and integrate over the reactor volume, V:

| Dϕ∇2ϕ'dV+1k'νϕνΣfϕ'dV-ϕΣa-δΣaϕ'dV=0 | ‎7.73 |
| --- | --- |

- we multiply Eq. of ‎7.3 by the poisoned flux ϕ' and perform the same volumetric integration to obtain:

| Dϕ'∇2ϕdV+1kϕ'νΣfϕdV-ϕ'ΣaϕdV=0 | ‎7.74 |
| --- | --- |

- Subtracting Eq. (7.74) from Eq. (7.73) then yields:

| Eqn. 7.74 - Eqn. 7.73 → Dϕ∇2ϕ'-ϕ'∇2ϕdV+1k'-1kννΣfϕϕ'dV-δΣaϕϕ'dV=0 | ‎7.75 |
| --- | --- |

- We prove that the first integral is zero by performing two calculations then subtracting them:

| ∇∙ϕ∇ϕ'=ϕ∇2ϕ'+(∇ϕ)∙∇ϕ' | ‎7.76 |
| --- | --- |

| ∇∙ϕ'∇ϕ=ϕ'∇2ϕ+(∇ϕ')∙∇ϕ | ‎7.77 |
| --- | --- |

- Subtract the two equations:

  - The second term of LHS cancels

  - This leaves integrand of the first integral

  - Replace this integrand by the RHS of the subtraction

  - Take the gradient as the common operator

  - Use divergence theorem: (∇∙ F)dV=ΩF.dA to replace volume integration (over the reactor body) to surface integration (over the reactor boundary surface):

  - The flux at the surface is set as zero as in the usual boundary condition:

  - Both fluxes,  ϕ &  ϕ', must vanish on the extrapolated surface A of the reactor:

| ϕ∇2ϕ'-ϕ'∇2ϕdV=∇∙ϕ∇2ϕ'-ϕ'∇2ϕdV=ϕ∇ϕ'-ϕ'∇ϕ∙dA=0 | ‎7.78 |
| --- | --- |

- With the first eliminated as shown, Eq. (‎7.75) is expressed as:

| 1k'-1kϕνΣfϕ'dV-ϕδΣaϕ'dV=0 | ‎7.79 |
| --- | --- |

- Suppose the reactor is initially critical, then k=1 and the reactivity following control insertion will be ρ=(k'-1)/k', 1k'-1k=1k'-11=1-k'k'=-ρ, the last numbered equation can be written as:

| ρ=-ϕδΣaϕ'dVϕνΣfϕ'dV | ‎7.80 |
| --- | --- |

- We have made no approximations in obtaining this equation. Moreover, if the added absorption is uniform over the core, the cross sections may be pulled outside the integrals, yielding for the reactivity,

- Take the added absorption to be uniform over the core, thus you could pull

| ρ=-δΣaΣf=-1k∞νΣfΣaδΣaνΣf=-δk∞ | ‎7.81 |
| --- | --- |

- If the control material is localized within some volume of the reactor, the distribution of the perturbed flux ϕ' must be treated explicitly. Provided the perturbation to the flux is small, however, we may write  ϕ'=ϕ+δϕ and ignore the size of δϕ relative to ϕ. The result is called the first-order perturbation approximation:

k∞=νΣfΣa

| ρ=-δΣaϕ2dVνΣfϕ2dV | ‎7.82 |
| --- | --- |

### Partially Inserted Control Rod

- Equation (7.82) serves as a basis for estimating the reactivity worth of a partially inserted control rod,

  - provided the reactivity associated with it is not large enough to significantly distort the flux distribution.

- Consider a uniform cylindrical reactor and begin by rewriting the numerator of Eq. (7.82) explicitly in cylindrical coordinates:

| ρ=-z=-H/2H/2ω=02πr=0RδΣaϕ2rdrdωdzνΣfϕ2dV | ‎7.83 |
| --- | --- |

- We evaluate the numerator only over that volume occupied by the control rod. Let  δΣa=Σac in that volume, and   δΣa=0 elsewhere.

- Assume that the rod is located at a distance r from the center of the core and is inserted into the core a distance x from its top as indicated in Fig. 7.7.

  - We continue to assume that the flux is a function only of r and z, and not of the azimuthal angle ω.

  - If the cross-sectional area of the rod, say, ac, is small we can ignore the r variation of the flux over its diameter. That is treating ϕ2, as constant for radial integration.

- The above expression approximately is:

| ρr,X=-acΣacz=H/2-xH/2ϕ2(r,z)dzνΣfϕ2(r,z)dV | ‎7.84 |
| --- | --- |

- For the uniform core with the flux described by Eq. (7.21, ϕr,z=C J02.405 r/Rcosπ z/H), we obtain

| ρr,X=-acΣacC2J022.450 r/ RνΣfϕ2(r,z)dV H/2-XH/2cos2π z/Hdz | ‎7.85 |
| --- | --- |

- Integration tables/programs:

  - Integrate[Cos[x]^2, x]   x/2 + 1/4 Sin[2 x]

  - cos2xdx=x/2+1/4sin(2x)

- The expression, 7.85, simplifies when normalized to a fully inserted control rod (x = H) at the same radial location. Dividing ρr,X by ρr,H we obtain:

| ρr,X=H/2-XH/2cos2π z/Hdz-H/2H/2cos2π z/Hdzρr,H=xH-12πsinπ x/Hρr,H | ‎7.86 |
| --- | --- |

- Figure 7.8 shows the control rod worth, that is, the negative reactivity created by the rod, normalized to the fully inserted rod.

- Note that the worth changes most rapidly when the tip is near the midplane of the core, where the flux is largest.

- Control rod worth also varies with the radial position of the core. For example, if we compare a rod situated at a radius r to one located along the core’s centerline, because J00=1, we obtain

| ρr,X=J022.450 r/ Rρ0,x | ‎7.87 |
| --- | --- |

### Control Rod Bank Insertion

- Normally, control rods are grouped into banks, which may be inserted or withdrawn in unison.

- Rod banks have large enough effects on reactivity and on the flux distribution that the perturbation technique described above is no longer applicable.

- To model the reactivity worth of a bank of control rods, we again consider a uniform cylindrical reactor, and specify a height to diameter ratio of one. To simplify the derivation, we take the origin of our r–z coordinates at the base of the reactor, as indicated in Fig. 7.9, and assume that the bank is inserted a distance x from the top of the reactor as indicated. Our starting point is therefore Eq. (7.4), but k∞ and M are no longer constants; they are functions of z, taking different values in the rodded and unrodded volumes of the core. Since the core is uniform in the radial direction, however, we may again employ separation of variables as in Eq. (7.11). Inserting these separated variables into Eq. (7.5) yields

| 1ψ1r∂∂r rddr ψ+1χ∂2∂z2χ-k∞z/k-1Mz2=0 | ‎7.88 |
| --- | --- |

- where we have replaced the diffusion with the migration length (L→M). The first term on the left must be a constant, since the remaining two terms vary only in z. We set it equal to -Br2 given by Eq. (7.19), thus satisfying Eq. (7.14) and meeting the boundary condition that ϕR,z=0. Thus Eq. (7.88) reduces to:

| d2dz2χ-k∞/k-1M2-Br2χ=0 | ‎7.89 |
| --- | --- |

- To account for the absorption in the control rod bank we subtract  δΣa from the infinite medium multiplication in the axial region of the core occupied by the rod bank. Hence for that region,

| k∞→k∞-δk∞=k∞1-ρb | ‎7.90 |
| --- | --- |

- where ρb=δk∞/k∞ is the reactivity worth of the bank when inserted the entire length of the core.

- For simplicity we assume that the presence of the control rods has no appreciable effect on the migration length. Equation (7.89) takes two forms for unrodded and rodded volumes (u and r, respectively). With the rod bank inserted to a depth x from the top of the core:

| d2dz2χu+α2χu=0, 0≤z≤H-x | ‎7.91 |
| --- | --- |

| d2dz2χr+(α2-β2)χr=0, H-x≤z≤H | ‎7.92 |
| --- | --- |

- Where:

| α2=1M2k∞k-1 -Br2 | ‎7.93 |
| --- | --- |

| β2=1M2k∞kρb | ‎7.94 |
| --- | --- |

- The solutions must meet the boundary conditions:

  - χu0=0

  - χuH=0

  - The interface condition between the rodded and unrodded regions at x:

  - The current also must match at x:

| χuH-x=χrH-x | ‎7.95 |
| --- | --- |

| ddzχuzH-x=ddzχrzH-x | ‎7.96 |
| --- | --- |

- where we have assumed that the diffusion coefficients in the rodded and unrodded core regions are the same.

- Solutions meeting the boundary conditions at z=0 and z=H may be shown to be:

  - sin0=0=sinπ

| χuz=Csinαz | ‎7.97 |
| --- | --- |

- and

| χrz=C'sinα2-β2H-z,α2>β2C'sinβ2-α2H-z,α2<β2 | ‎7.98 |
| --- | --- |

- Application of the interface conditions, Eqs. (7.95) and (7.96), leads to two additional equations relating C and C’.

  - χuH-x=χrH-x→CsinH-x=C'sinα2-β2H-x,α2>β2C'sinhβ2-α2H-x,α2<β2

- ddzχuzH-x=ddzχrzH-x→ C α cosH-x=α2-β2C'cosα2-β2H-x,α2>β2β2-α2C'coshβ2-α2H-x,α2<β2

- Taking their ratio of the two equations yields the transcendental equation

| αcotαH-x=-α2-β2cotα2-β2x,α2>β2 | ‎7.99 |
| --- | --- |

- or

| αcotαH-x=-α2-β2cothβ2-α2x,α2<β2 | ‎7.100 |
| --- | --- |

- Given the reactor’s material properties, along with the definitions of α and   β, these transcendental equations can be solved numerically for k.

- The two limiting values of k reduce to be what is expected. For the rods out case, Eq. (7.97) is applicable over the entire core. Thus, we must have  χuH=0, from which it follows that αu2=π/H2=Bz2, so that Eq. (7.93) yields

| ku=k∞1+M2Br2+Bz2 | ‎7.101 |
| --- | --- |

- Similarly, with the control rods fully inserted, Eq. (7.98) holds over the entire core, and we must have: χr0=0

- Only the first of the pair can meet this condition, and we obtain αr2-βr2=π/H2=Bz2, from which Eqs. (7.93) and (7.94) yield kr=1-ρrku. Next, we consider situations where the rod bank is partially inserted to a distance x.

- Figure 7.10 is a plot of the reactivity versus insertion length for two cores, both with height to diameter ratios of one, and both with rod bank worths of ρb=0.02. The ratio of core dimension to migration length, H/M, is central to understanding the reactivity curves and flux distributions. A core with a smaller value of H/M is referred to as neutronically tightly coupled, because disturbances, such as control rod bank insertions, travel across it in relatively few migration lengths, and therefore over relatively few neutron generations. In a loosely coupled core—one with a large H/M ratio—the converse is true: Many neutron generations are required to propagate a disturbance across the reactor. The net effect is that in the more tightly coupled core distribution of the neutron flux deviates less from that of a uniform core, and the pattern of control rod worth is closer to that of Eq. (7.86); this can be observed by comparing the H/M=10  curve to Fig. 7.8, which assumes no disturbance to the flux distribution. In contrast, the H/M=50 curve is quite skewed; the rod bank has little effect until it is inserted more than halfway into the core.

- The effects on flux distribution are also pronounced. As Fig. 7.11a indicates, as the rod bank is inserted into the more tightly coupled core, with H/M=10, the perturbed flux does not deviate greatly from the standard axial cosine distribution. For the more loosely coupled core, however, the flux is pushed toward the bottom of the core as the rods are inserted. This is illustrated for the ~ H/M = 50 case shown in Fig. 7.11b. Note that all the curves in Figs. 7.11a and 7.11b are normalized to the same average flux, thus illustrating that the flux peaking can become quite severe in the more loosely coupled core.

- A reactor’s power density (i.e., power per unit volume) is limited by thermal-hydraulic considerations. Thus, for a given class of reactors, higher powers imply larger volumes, and therefore larger values of H/M, and more loosely coupled cores.

- Comparing Tables 4.1 and 7.1 we see that both achievable average power densities and migration lengths vary greatly between reactor designs. Generally, thermal reactors are more loosely coupled than fast reactors, and they may have values of H/Msubstantially exceeding 50. Thus prevention of excessive power peaking from control rod movements, refueling patterning, and other phenomena becomes an increasing concern. In the following chapter we take up the coupling between neutronics and the removal of heat from power reactors that bears heavily on these issues.

## Problems

Let us take the diffusion length to be a particular value: say L=4.3cm

(PWR, L.T=2.85cm, L.F=5.10, M=5.84)

H=2R=D

B2=2.405/R2+π/H2

k=k∞1+M2B2→1=1.021+4.32B2→B=0.0328887

B2=0.03288872=2.405*2/D2+π/D2→D= 174.682cm

V=πD22D=π4D3=4.345m3

Sphere:

B=0.0328887

1ϕ1r2ddr r2ddr ϕ+B2=0→ϕ=C*CosπR*r

B2=π/R2→R= 95.522cm

V=43πR3=3.65 m3

Define the optimization parameter x=H/D

Rewrite:

B2=2.405*2/D2+π/xD2

V=πD22H=π4D3x→D=4Vxπ1/3

Eliminate D and fix volume V:

ddxB2=0→x=HD=0.923

| Partial Differential Equation | Graphical Solution |
| --- | --- |
| ∂2∂x2ϕx+A2ϕx=0 solution = DSolveValue[y''[x] + A^2*y[x] == 0, y[x], x] solutionTable1 = Table[solution /. {C[1] -> 0, C[2] -> j}, {j, -2, 2}] solutionTable2 = Table[solution /. {C[1] -> j, C[2] -> 0}, {j, -2, 2}] A = 3; a = 1; b = 2; Row[{ Plot[solutionTable1, {x, -a, b}, ImageSize -> Medium] , Plot[solutionTable2, {x, -a, b}, ImageSize -> Medium] }] | ϕx=C1 CosA x+ C2 SinA x {-2 Sin[3 x], -Sin[3 x], 0, Sin[3 x], 2 Sin[3 x]} {-2 Cos[3 x], -Cos[3 x], 0, Cos[3 x], 2 Cos[3 x]} |
| ∂2∂x2ϕx - A2ϕx=0 “Helmholtz Equation” Clear[A, y, x]; solution = DSolveValue[y''[x] - A^2*y[x] == 0, y[x], x] solutionTable1 = Table[solution /. {C[1] -> 0, C[2] -> j}, {j, -2, 2}] solutionTable2 = Table[solution /. {C[1] -> j, C[2] -> 0}, {j, -2, 2}] A = 3; a = 1; b = 2; Row[{ Plot[solutionTable1, {x, -a, b}, ImageSize -> Medium] , Plot[solutionTable2, {x, -a, b}, ImageSize -> Medium] }] | ϕx=C1 ExpA x+ C2 Exp-A x {-2 E^(-A x), -E^(-A x), 0, E^(-A x), 2 E^(-A x)} {-2 E^(A x), -E^(A x), 0, E^(A x), 2 E^(A x)} |
| x2∂2∂x2ϕx+x∂∂xϕx-x2-n2ϕx=0 ” Clear[A, y, x]; solution = DSolveValue[y''[x] - A^2*y[x] == 0, y[x], x] solutionTable1 = Table[solution /. {C[1] -> 0, C[2] -> j}, {j, -2, 2}] solutionTable2 = Table[solution /. {C[1] -> j, C[2] -> 0}, {j, -2, 2}] A = 3; a = 1; b = 2; Row[{ Plot[solutionTable1, {x, -a, b}, ImageSize -> Medium] , Plot[solutionTable2, {x, -a, b}, ImageSize -> Medium] }] | ϕx=C1 Jn,x+ C2 Yn, x ϕx=C1J0,x+C2Y0, x=C1J0x+C2Y0x ϕx=C1J1,x+C2Y1, x=C1J1x+C2Y1x |
| Series[BesselJ[0, x], {x, 0, 10}] Series[BesselJ[1, x], {x, 0, 10}] Series[Sin[x]/x, {x, 0, 10}] | J0,x=1-x24+x464-x62304+x8147456-x1014745600+Ox11 J1,x=x2-x316+x5384-x718432+x91474560+Ox11 1-x26+x4120-x65040+x8362880-x1039916800+Ox11 |

# Summary Table:

| # | Summary Equations | Figures |
| --- | --- | --- |
| 7.3 | Time-Independent Diffusion Equation (Eqn 7.1): ∇.D∇ϕr+νΣfϕr-Σaϕr=0→1ψ1r∂∂r rddr ψ+1χ∂2∂z2χ+B2=0 k=k∞1+L2B2 k∞=νΣfΣa B2=2.405/R2+π/H2 ϕr,z=C J02.405 r/Rcosπ z/H ϕr,z=3.63PγΣfVJ02.405rRcosπzH | Uniform reactors: |
| 7.4 | Move from thermal diffusion lengths to migration length: L1=D1/Σr L2=D2/Σa → M2=L12+L22 k=11+M2B2k∞ | Neutron Leakage |
| 7.5 |  | Reflected Reactors |
| 7.6 |  | Control Poisons |

https://physics.stackexchange.com/questions/607964/what-is-fermi-age-what-does-it-represent

The constant is assumed positive which gives: ∂2∂z2χz+Bz2χz=0→χz=C1sinBzz+C2cosBzz